| Appx A (B Veh) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FMN READINESS STATE : 'B' VEH OF HQ 3 CORPS (EME) FOR THE MONTH OF Oct 2025 |  |  |  |  |  |  |  |  |  |  |
| Ser No | Category (Make & Type) | Dependency |  |  |  |  |  | FMC | Remarks (To incl present loc of eqpt EOA) |  |
|  |  | Auth (UE) | Held (UH) | MUA | OH | R4 | Total |  |  |  |

This table is a standard military equipment readiness report, specifically for "B" Vehicles (unarmored transport vehicles like trucks, jeeps, and ambulances) within the Corps of Electronics and Mechanical Engineers (EME).

Here is the breakdown of the column names and their specific meanings within the defense context:

Column Definitions

| Column Name | Full Form / Detail | Description |
| --- | --- | --- |
| Ser No | Serial Number | The numerical order of the entries in the list. |
| Category (Make & Type) | Vehicle Category | Specifies the manufacturer (e.g., Tata, Ashok Leyland) and the specific model or capacity (e.g., 2.5 Ton, 4x4, Gypsy). |
| Dependency: Auth (UE) | Authorized (Unit Establishment) | The number of vehicles the unit is officially permitted to have according to its organizational structure. |
| Dependency: Held (UH) | Held (Unit Holding) | The actual number of vehicles currently physically present or assigned to the unit's inventory. |
| MUA | Minor Under Activity | Vehicles undergoing minor repairs or routine maintenance that can be finished quickly at the unit level. |
| OH | Overhaul | Vehicles undergoing major strip-down repairs or reconditioning, usually at a central workshop or depot. |
| R4 | Repair Level 4 | Refers to heavy repairs that cannot be done at the unit level and require specialized field or base workshops. |
| Total (Under Repair) | Total Non-Mission Capable | The sum of MUA + OH + R4; the total number of vehicles currently out of service. |
| FMC | Fully Mission Capable | The number of vehicles that are 100% fit for immediate operational deployment. |
| Remarks | Observations / Location | Specific notes, including the EOA (Equipment On Account) or the current physical location of the asset. |

Key Contextual Terms

- FMN (Formation): Refers to a large military organization (like 3 Corps).

- 'B' Veh: Soft-skinned (unarmored) vehicles used for logistics and transport.

- HQ 3 CORPS (EME): The headquarters of the 3rd Corps' maintenance and engineering branch, responsible for the upkeep of these assets.

| Appx A (C Veh) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FMN READINESS STATE : C VEHS |  |  |  |  |  |  |  |  |  |
| Ser No | Category (Make & Type) | Dependency |  | NMC |  |  | Total NMC (Nos) | FMC | Remarks (Demand details of VOR/EOA ) |
|  |  | Auth (UE) | Held (UH) | MUA | OH | R4 |  |  |  |

This table is a readiness report specifically for "C" Vehicles, which typically refers to Engineering Equipment and earth-moving machinery (e.g., dozers, excavators, cranes).

Below is the extraction of the column names and their military operational context:

Column Definitions

| Column Name | Full Form / Detail | Military Domain Description |
| --- | --- | --- |
| Ser No | Serial Number | The sequential numbering of equipment entries. |
| Category (Make & Type) | Equipment Category | The specific brand and functional model of the engineering vehicle. |
| Dependency: Auth (UE) | Authorized (Unit Establishment) | The number of vehicles officially allotted to the unit by organization tables. |
| Dependency: Held (UH) | Held (Unit Holding) | The actual quantity of equipment currently in the unit's possession. |
| NMC: MUA | Non-Mission Capable: Minor Under Activity | Vehicles undergoing light maintenance or minor repairs at the unit level. |
| NMC: OH | Non-Mission Capable: Overhaul | Vehicles currently stripped down for major lifecycle restoration or reconditioning. |
| NMC: R4 | Non-Mission Capable: Repair Level 4 | Equipment requiring heavy, complex repairs at a specialized base workshop. |
| Total NMC (Nos) | Total Non-Mission Capable | The total number of units currently unserviceable (Sum of MUA + OH + R4). |
| FMC | Fully Mission Capable | The number of engineering vehicles ready for immediate operational tasks. |
| Remarks | Details of VOR/EOA | Notes on Vehicle Off Road (VOR) status or Equipment On Account (EOA) details. |

Key Contextual Terms

- FMN Readiness State: The operational preparedness level of the specific Formation.

- 'C' Vehs: Heavy engineering and construction plant equipment.

- VOR (Vehicle Off Road): A critical status indicating the vehicle is grounded due to lack of spares or major fault.

| FMN READINESS STATE : ARMT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ser | Make & Eqpt | Dependency |  | NMC Due to |  |  |  |  |  |  |  | Total NMC (Nos) | PMC (Nos) (Due to OH) | FMC | NMC % | PMC% | FMC % | Avl % | Remarks (To incl present loc of eqpt EOA) |
|  |  | Auth (UE) | Held (UH) | Eng/ Brl | MUA | Spares | OH | MR | FR | R4 | OBE |  |  |  |  |  |  |  |  |

The provided header is for a Formation (FMN) Readiness State report specifically for Armament (ARMT), which includes weaponry such as small arms, artillery, and tank guns.

Below is the extraction of the column names and their specific military technical meanings:

Column Definitions (ARMT Readiness)

| Column Name | Full Form / Detail | Military Domain Description |
| --- | --- | --- |
| Ser | Serial Number | Sequential list entry. |
| Make & Eqpt | Manufacturer & Equipment | The specific model and nomenclature of the weapon/armament (e.g., 5.56mm INSAS, 155mm Bofors). |
| Dependency: Auth (UE) | Authorized (Unit Establishment) | The official quantity of the armament the unit is scaled to hold. |
| Dependency: Held (UH) | Held (Unit Holding) | The actual physical quantity of the armament currently on the unit's books. |
| NMC Due to: Eng/Brl | Engineering / Barrel | Non-Mission Capable (unserviceable) due to barrel wear, life expiry, or major engineering faults. |
| NMC Due to: MUA | Minor Under Activity | Unserviceable due to minor repairs being carried out locally. |
| NMC Due to: Spares | Lack of Spares | Grounded/Unserviceable specifically because replacement parts are unavailable. |
| NMC Due to: OH | Overhaul | Sent for complete reconditioning at a base or regional workshop. |
| NMC Due to: MR | Medium Repair | Intermediate level repairs that exceed unit capability but aren't full overhauls. |
| NMC Due to: FR | Field Repair | Repairs being conducted by a Field Workshop in the forward area. |
| NMC Due to: R4 | Repair Level 4 | Heavy repairs requiring the highest level of workshop facilities. |
| NMC Due to: OBE | Out of Bounds / Beyond Economic Repair | Equipment that is likely to be scrapped or is awaiting a decision on its disposal. |
| Total NMC (Nos) | Total Non-Mission Capable | The total count of unserviceable weapons. |
| PMC (Nos) | Partially Mission Capable | Equipment that can still function but with limited secondary features or degraded performance. |
| FMC | Fully Mission Capable | The number of weapons 100% fit for combat. |
| NMC % | Non-Mission Capable Percentage | The percentage of the inventory that is currently unserviceable. |
| PMC % | Partially Mission Capable % | The percentage of the inventory with limited capability. |
| FMC % | Fully Mission Capable Percentage | The percentage of the inventory ready for immediate operational use. |
| Avl % | Availability Percentage | The total percentage of equipment (FMC + PMC) available for use. |
| Remarks | Present Location / EOA | Notes including the current physical location or Equipment On Account (EOA) details. |

Analysis of Technical Formulas

In this report, the following logic is typically used to calculate readiness:

- Total NMC (Nos) = Eng/Brl+MUA+Spares+OH+MR+FR+R4+OBE.

- FMC % = (Held(UH)FMC)×100.

| FMN READINESS STATE : SMALL ARMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ser | Make & Eqpt | Dependency |  | NMC Due to |  |  |  |  | Total NMC (Nos) | PMC (Nos) | FMC (Nos) | NMC % | PMC% | FMC % | Avl % | Remarks (To incl present loc of eqpt EOA) |
|  |  | Auth (UE) | Held (UH) | Eng/ Brl | MUA | Spares | Under OH /MR/R4/ FR/ Base Repair | OBE |  |  |  |  |  |  |  |  |

This extraction covers the final header for Small Arms readiness, which tracks individual and crew-served weaponry like rifles, carbines, and machine guns.

Column Definitions (Small Arms)

| Column Name | Full Form / Detail | Military Domain Description |
| --- | --- | --- |
| Ser | Serial Number | The numerical order of weapons listed. |
| Make & Eqpt | Make & Equipment | The specific nomenclature of the firearm (e.g., 7.62mm SLR, AK-47, Glock 17). |
| Dependency: Auth (UE) | Authorized (Unit Establishment) | The quantity of weapons the unit is officially entitled to hold based on its strength. |
| Dependency: Held (UH) | Held (Unit Holding) | The total number of weapons currently in the unit's inventory or "on the books." |
| NMC Due to: Eng/Brl | Engineering / Barrel | Weapons unserviceable due to worn-out barrels (smooth rifling) or major structural engineering failures. |
| NMC Due to: MUA | Minor Under Activity | Arms undergoing light maintenance or quick repairs at the unit armory. |
| NMC Due to: Spares | Lack of Spares | Weapons sidelined because critical replacement parts (pins, springs, etc.) are not available. |
| NMC Due to: Under OH / MR / R4 / FR / Base Repair | Repair Pipeline | A consolidated category for weapons sent to higher-level workshops for Overhaul, Medium, or heavy Base repairs. |
| NMC Due to: OBE | Out of Bounds / Beyond Economic Repair | Weapons that are unsafe or too costly to fix and are categorized for condemnation/scrapping. |
| Total NMC (Nos) | Total Non-Mission Capable | The total count of unserviceable weapons (Sum of the "NMC Due to" sub-columns). |
| PMC (Nos) | Partially Mission Capable | Weapons that are functional but have minor defects that don't prevent use (e.g., damaged sights or furniture). |
| FMC (Nos) | Fully Mission Capable | The count of weapons that are 100% operational and battle-ready. |
| NMC % | Non-Mission Capable % | Percentage of total holdings that are currently unserviceable. |
| PMC % | Partially Mission Capable % | Percentage of holdings that are partially operational. |
| FMC % | Fully Mission Capable % | The key readiness metric; the percentage of total arms ready for immediate combat. |
| Avl % | Availability Percentage | The combined percentage of functional weapons (FMC+PMC). |
| Remarks | Present Location / EOA | Notes on where the weapons are physically located or their status On Account (EOA). |

Comparison of Repair Categories

You may notice that Small Arms uses a consolidated column for major repairs compared to the ARMT table. This is because small arms are often "swapped" or sent in bulk for base repairs rather than being tracked individually through every single field repair stage like a large artillery piece would be.

| FMN READINESS STATE : INSTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ser | Make & Eqpt |  | Dependency |  | NMC due to |  |  | Total NMC (Nos) | PMC (Nos) | FMC (Nos) | NMC % | PMC% | FMC % | Avl % | Remarks (To incl present loc of eqpt EOA) |
|  |  |  | Auth (UE) | Held (UH) | Spares | Under OH /R4/FR/ Base Repair | OBE |  |  |  |  |  |  |  |  |

Based on the provided document headers, here is the detailed extraction for the FMN Readiness State: INSTS (Instruments) report. This table is typically used to track the serviceability of optical, electronic, and surveying instruments such as binoculars, night vision devices, and laser rangefinders.

FMN Readiness State: INSTS (Instruments)

| Column Name | Full Detail | Military Domain Description |
| --- | --- | --- |
| Ser | Serial Number | The sequential order of the instrument entries. |
| Make & Eqpt | Make & Equipment | The specific nomenclature and manufacturer of the instrument (e.g., Night Vision Goggles, Laser Range Finder). |
| Dependency: Auth (UE) | Authorized (Unit Establishment) | The official quantity of instruments the unit is authorized to hold according to its organizational structure. |
| Dependency: Held (UH) | Held (Unit Holding) | The actual quantity of instruments currently in the unit's possession or inventory. |
| NMC due to: Spares | Non-Mission Capable (Spares) | Instruments that are unserviceable specifically because required replacement parts or components are unavailable. |
| NMC due to: Under OH /R4/FR/ Base Repair | Repair Pipeline | Instruments currently undergoing various levels of repair: Overhaul (OH), Level 4 (R4), Field Repair (FR), or Base Workshop repair. |
| NMC due to: OBE | Out of Bounds / Beyond Economic Repair | Equipment that is no longer serviceable and is considered too costly or impossible to repair; often awaiting disposal. |
| Total NMC (Nos) | Total Non-Mission Capable | The total count of instruments that are currently unserviceable (sum of the "NMC due to" columns). |
| PMC (Nos) | Partially Mission Capable | Instruments that are functional but have minor defects that do not entirely prevent their primary use. |
| FMC (Nos) | Fully Mission Capable | The number of instruments that are 100% operational and ready for immediate deployment. |
| NMC % | Non-Mission Capable Percentage | The percentage of the total inventory that is currently unserviceable. |
| PMC % | Partially Mission Capable Percentage | The percentage of the inventory that has limited or partial functionality. |
| FMC % | Fully Mission Capable Percentage | The key readiness metric representing the percentage of instruments ready for mission use. |
| Avl % | Availability Percentage | The combined percentage of equipment (FMC + PMC) available for operational tasks. |
| Remarks | Present Loc / EOA | Specific notes, including the current physical location of the equipment or its status on the Equipment On Account (EOA). |

| STATE OF CBRN EQPT TO BE SUSTAINED BY EME |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ser No | Make & Type | Dependency |  | UNSV |  |  |  |  | UNSV | ‘S’ | Remarks |  |
|  |  | Auth | Held | MUA | Spares | DRWO | FR | DR Raised |  |  |  |  |

The State of CBRN EQPT table tracks equipment specifically designed for Chemical, Biological, Radiological, and Nuclear defense that must be sustained by the EME (Electronics and Mechanical Engineers).

Below is the full list of columns and their detailed descriptions within the military domain:

CBRN Equipment State Table

| Column Name | Full Detail | Description |
| --- | --- | --- |
| Ser No | Serial Number | The sequential order of the items listed. |
| Make & Type | Nomenclature | The specific manufacturer and model of the CBRN gear (e.g., Gas Masks, Decontamination Kits, Dosimeters). |
| Dependency: Auth | Authorized | The official quantity of equipment the unit is mandated to hold according to its establishment. |
| Dependency: Held | Held | The actual quantity of equipment currently in the unit’s inventory. |
| UNSV: MUA | Minor Under Activity | Items that are unserviceable due to minor faults being repaired at the unit level. |
| UNSV: Spares | Lack of Spares | Equipment that cannot be used because specific replacement parts are currently unavailable. |
| UNSV: DRWO | District/Divisional Repair Workshop Order | Equipment that has been formally sent to a workshop for repair with an active job order. |
| UNSV: FR | Field Repair | Equipment undergoing repairs in the forward area by a Field Workshop. |
| UNSV: DR Raised | Demand Raised | Indicates that a formal demand for the repair or replacement of the unserviceable item has been initiated. |
| UNSV (Total) | Total Unserviceable | The total count of equipment that is currently not fit for use. |
| ‘S’ | Serviceable | The total number of units that are fully functional and mission-ready. |
| Remarks | Observations | Any additional context, such as the specific location of the gear or estimated date of repair completion. |

Export to Sheets

Key Acronyms in this Context

- CBRN: Chemical, Biological, Radiological, and Nuclear.

- EME: Corps of Electronics and Mechanical Engineers (responsible for maintenance).

- UNSV: Unserviceable (the equipment is broken or degraded).